Document:

Working definitions of

Water scarcity and Drought

Version: 4.0

Date: 27 April 2012

Prepared by / compiled by:

Guido Schmidt (1), Juan José Benítez (1), and Carlos Benítez (1), with contributions from Rafael Seiz-Puyuelo (2),  José María Hernández-Torres (2) and the Expert Group on Water Scarcity and Droughts of the Common Implementation Strategy for the Water Framework Directive

Consultants: INTECSA-INARSA, S.A. (1), based on a previous draft by TYPSA (2)

Version History

| Version | Date | Author | Status and description | Distribution |
| --- | --- | --- | --- | --- |
| 1.0 | 13/06/2011 | TYPSA | Draft version prepared by TYPSA for review. Inclusion of additional comments by Jürgen Vogt (JRC). Inclusion of complementary quotes. Inclusion of additional comments and quotes | EG WS&D |
| 2.0 | 29/03/2012 | Intecsa-Inarsa | Inclusion of additional comments and quotes after the consultation of the EG and after the presentation at the EG meeting in October 2011 | EG WS&D Leaders |
| 3.0 | 12/04/2012 | Intecsa-Inarsa | Inclusion of comments from the EG Leaders. Version 3.1 for discussion at the EG meeting, with complementary comments from FR and ES. | EG WS&D |
| 4.0 | 27/04/2012 | Intecsa-Inarsa | Including final comments from the Expert Group on Water Scarcity & Droughts |  |

Disclaimer: The sole responsibility for the content of this report lies with the authors. It does not necessarily reflect the opinion of the European Commission. The European Commission is not responsible for any use that may be made of the information contained therein.

Contents

1.	Aim	3

2.	Currently used definitions	3

2.1.	EU and CIS Documents	3

2.2.	Other sources of information	3

2.3.	Emerging problems	3

3.	Key concepts	3

3.1.	Similarities and differences between drought and water scarcity	3

4.	Proposed working definitions	3

4.1.	Drought definition	3

4.2.	Water scarcity definition	3

4.3.	Complementary text	3

5.	References	3

# Aim

The aim of this short report is to review the different definitions and descriptions of water scarcity and drought phenomena endorsed by the Water Directors in 2006 or being used by the European Commission, other European institutions or by the Member States when referring to drought and water scarcity, and to propose a clearer “working definition” to be used onwards in the policy context.

This document does not deal with the definitions of prolonged droughts (WFD and CIS, 2009a), or other operational drought concepts (the sequence of meteorological, agricultural and hydrological drought).

# Currently used definitions

The main definitions used at the EU level for droughts and water scarcity are included in the following documents.

## EU and CIS Documents

- The Policy summary on Water Scarcity and Droughts endorsed at the Informal meeting of Water Directors of the European Union, Candidate and EFTA Countries in Salzburg (1 and 2 June 2006). In terms of definitions, it does not provide specific ones for the different phenomena, but the following aspects can be extracted:

“…it is important to distinguish between imbalances and aridity. Imbalances arise when water demands exceed the supply capacity of the natural system. Aridity, which is a natural phenomenon, generally describes low water availability in an ecosystem due to low precipitation and/or high evaporation rates. It is equally important to differentiate between aridity as a long-term average feature and a drought situation that indicates a deviation from the average situation, but is still within the ecosystem natural variability. It is also necessary to differentiate among transitory periods of water deficiency, caused by exceptional droughts and long-term imbalances of available water resources and demands. Both phenomena require different sort of measures and actions but including both, as minimum common denominator, water demand management measures”

- This document is the summary of the previous MED Joint Process WFD/EUWI Water Scarcity Drafting Group (2006:13-24) that includes further detailed and reasoned definitions:

- “...Drought is a normal, recurrent feature of climate, although often erroneously considered an unexpected and extraordinary event. It occurs in virtually all climatic zones, but its characteristics vary significantly from one region to another. Drought is a temporary aberration within the natural variability and can be considered an insidious hazard of nature; it differs from aridity which is a long-term, average feature of climate...

- Droughts generally result from a combination of natural factors that can be enhanced by anthropogenic influences. The primary cause of any drought is a deficiency in rainfall, and, in particular, the timing, distribution, and intensity of this deficiency in relation to the existing water storage, demand, and use. This deficiency can result in a shortage of water necessary for the functioning of a natural (eco-)system, and / or necessary for a certain human activities…”

- Water scarcity is “…An imbalance in water supply and demand is a situation where there is insufficient water to satisfy long-term average requirements is generally considered as water scarcity… Nevertheless defining scarcity for policy-making purposes is very difficult. The term ‘water scarcity’ has the following specific meanings: an imbalance of supply and demand under prevailing institutional arrangements and/or prices, an excess of demand over available supply, and/or a high rate of utilization compared to available supply, especially if the remaining supply potentials are difficult or costly to tap…”

- The 2007 Communication on Water Scarcity & Droughts details “…while ‘drought’ means a temporary decrease in water availability due for instance to rainfall deficiency, ‘water scarcity’ means that water demand exceeds the water resources exploitable under sustainable conditions…”.

- The CIS Drought Management Plan Report follows the definitions of the 2007 Communication, and considers additionally that “…while the terms ‘water scarcity’ and ‘drought’ are commonly used interchangeably, they are quite different phenomena affected [by] water management practices and natural causes…” (CIS, 2007:5)

- Respectively, the CIS Guidance Document on exemptions to the environmental objectives defines that “…a drought - contrary to water scarcity - is a natural unpredictable phenomenon. The appearance of a drought is not generated by human activities. However, the impacts of a drought episode may be exacerbated by mismanagement practices...” (CIS, 2009a:24).

- The CIS Guidance Document on River basin management in a changing climate defines:

“...The term “drought” refers to a temporary deviation from long-term average or normal conditions in a hydrological context with regard to water supply. It usually originates in a considerable reduction in precipitation over a significant period of time and with a substantial spatial extent. Through interconnection within the natural water cycle, changes in other climate or land surface conditions can also cause droughts, e.g. through an increase in ambient air temperature and consequently higher evapotranspiration rates. The propagation of droughts through the water cycle may cause temporary shortages in water supply; their impacts depend on vulnerability and coping capacities on the demand side. Climate change can affect the gradual change of average conditions, as well as the frequency and magnitude of deviations from them, thus affecting the occurrence of drought events.

The term “water scarcity” indicates a long-term imbalance between water supply and demand in a region (or in a water supply system) possibly characterized by a semi-arid or arid climate and/or enhanced by a fast increase of water demand, associated with population growth and/or an extension of irrigated agriculture. Climate change may create or intensify water scarcity problems in a region, either through a reduction in water supply or through an increase in water demand...” (CIS, 2009b:93).

## Other sources of information

- The main website for “Water scarcity and droughts” of the European Commission includes separate definitions and additional information on causes, indicators and forecasts, including a set of maps. The definition given for droughts is similar to the text of the 2007 Communication and regarding water scarcity, it “…occurs where there are insufficient water resources to satisfy long-term average requirements. It refers to long-term water imbalances, combining low water availability with a level of water demand exceeding the supply capacity of the natural system.”

Furthermore, the European Drought Observatory (EDO) highlights that “Drought is one of the major weather related disasters. Persisting over months or years, it can affect large areas and may have serious environmental, social and economic impacts. These impacts depend on the duration, severity and spatial extent of the precipitation deficit, but also and to a large extent on the environmental and socio-economic vulnerability of affected regions”.

Following the European Environmental Agency, “...drought results from a combination of meteorological, physical and human factors. The primarily cause of any drought is a deficiency in rainfall...human factors include demand for water...” (EEA, 2001:8). Water scarcity is associated to overexploitation, and “...droughts are distinct from water scarcity, being a natural phenomenon defined as a sustained and extensive occurrence of below-average water availability...” (EEA, 2009:9).

- Regarding droughts, the 2007 Xerochore research project considers that weather phenomenon produces a progressive set of consequences in various areas, such as lower yields of rainfed crops, reduced contributions to surface water flows and groundwater recharge, supply difficulties of urban water demands and other uses, etc. As a result, water experts classify several types of drought: meteorological, agricultural, hydrological and socioeconomic.

## Emerging problems

A number of problems due to the definitions can be identified as follows:

- Length and detail: very detailed definitions vs. very short texts.

- Contradictions and overlaps (e.g. due for instance to rainfall deficiency; and regarding the duration of the events).

- Key difference related either to the temporary duration (e.g. temporary vs. long-term) or the causes (natural phenomenon vs. imbalances). Furthermore, some definitions remark the imbalance from the excess of demand side and others do it from the insufficiency to satisfy requirements angle.

- Unclear explanation of the interrelationships between the different events.

# Key concepts

The terms water scarcity and drought are complex. There is no scientific agreement about their precise definition, in particular regarding drought; and due to the very similar effects of both events, water managers, public and media use both terms often in an indistinct manner. Moreover, different Member States also use different concepts and definitions of water scarcity and drought according to their particular circumstances and needs of developing measures in response to the problems of lack of water derived from these situations.

From the policy point of view, and in particular in order to define adequate policy responses, it is necessary to make a distinction between droughts and water scarcity based on their causes and the opportunities for action. Therefore, the following Fig. 1 should be used as a baseline to frame these terms for policy making at European level:

| - Timescale and causes of water scarcity, drought and related concepts |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Timescale |  |  |
|  |  | Short-term (days, weeks) | Mid-term (months, seasons, years) | Long-term (decades) |
| Causes | Natural | Dry Spell | Drought | Aridity |
|  | Man-made | Water shortage | Water scarcity | Desertification |

Thus, drought is the term to define natural events that last mid-term, and water scarcity is used for man-made situations in the mid-term. Those are the key ones for EU policy action, and the actions should also affect positively the long-term concepts. The short-term concepts will be addressed by (water) management actions of the competent authorities. Furthermore, bear in mind that aridity and desertification cannot be defined merely in terms of a longer duration of drought events or water scarcity.

## Similarities and differences between drought and water scarcity

Furthermore, there are similarities and differences between drought and water scarcity. They can be defined in the following way for the EU policy purposes:

- Key elements for the definition of water scarcity and drought

| Aspect | Drought | Water scarcity |
| --- | --- | --- |
| Causes | Natural, due to a reduction of precipitation over a certain time period. High Temperatures, strong winds, low relative humidity, timing (onset, duration and end) as well as characteristics of rain can increase the severity. | Man-made, due to an overuse of water resources, caused by consumption becoming significantly higher than the natural renewable availability, or their pollution (reducing their suitability for water uses). |
| Occurrence | Drought is a normal, recurrent feature of all climates and can happen in all parts of Europe.. Its occurrence can be related to large-scale changes in atmospheric circulation patterns. | Due to the increase of water consumption, water scarcity is increasingly relevant and recurrent across Europe. |
| Duration | Droughts are very variable in their duration. They can last from a few weeks to several years | Usually, water scarcity is characterised bya permanent and continued degradation of water ecosystems and less water availability for other (economic) functions |
| Impacts | Very variable according to occurrence, severity and duration of the event as well as the sensitivity of affected ecosystems, economy and society and influenced also by the relative humidity of soils, capacity of storage for groundwater and streamflow of surface waters. When occurring in already water scarce affected areas, droughts shall have the most severe effects If water scarcity and drought pass certain thresholds, they can significantly affect the environment (terrestrial and freshwater ecosystems, air, soils, salt intrusion), the economy (agriculture and water uses) and society (e.g. urban water shortages, welfare, recreational activities, cultural and aesthetic concerns) |  |
| Spatial extent | Regarding the geographical extension, droughts and water scarcity can happen at local level or cover entire RBDs. Drought events are even reported for wide areas of the EU. |  |
| Predictability | Drought forecasting is currently based both on statistical analysis of the historical occurrences and numerical weather forecast, with a timeframe from a few days up to several months (with increasing uncertainty). Its implementation is planned in the frame of EDO | Water scarcity is predictable for the mid- and long-term in the frame of RBMPs if adequate information on water availability and consumption and trends has been compiled, and considering the uncertainty e.g. of climate change predictions |
| Interaction | When droughts occurr in an area characterised by water scarcity, their impact will be more severe, as they are more vulnerable. Heat waves can aggravate droughts and water scarcity situations. Water scarcity can also be an effect of overexploitation due to (concurrent) drought events, but this does not apply vice versa (drought is not an effect of water scarcity) |  |
| Environmental thresholds | (Freshwater) ecosystems are often characterised by and adapted to recurrent natural variations in precipitation and streamflow. Nonetheless, exceptionally severe droughts -or the combined impact of droughts with man-made overabstraction/ water scarcity can result in irreversible changes in the ecosystems | Water scarcity usually affects the ecological status of ecosystems, depending on its duration, relevance and the sensitivity of the ecosystem (incl. functions and elements) |
| Costs | In general, few data are available about the precise costs of water scarcity or drought situations (for marketable sectors, like agriculture, energy,…). If data are available, they should evaluate the combined effects of a drought and geographically overlapping water-scarce area |  |
| Public Awareness Indicators | The Standard Precipitation Index (SPI) reflects temporal deviations of rainfall with respect to the statistically expected rainfall derived from a reference period The Fraction of Absorbed Photosynthetically Active Radiation (fAPAR) reflects deviations in the fraction of solar energy absorbed by vegetation canopy respect to the statistically expected from a reference period | The Water Exploitation Index (WEI+) reflects the relation between water availability and abstraction/consumption (still in testing phase) |
| Possible measures to prevent or mitigate effects | Drought forecasting, risk prevention (e.g. insurances, climate change adaptation, increased flexibility of water usage, increase water efficiency, protection of vulnerable species and habitats, governance rules for different drought thresholds) and emergency actions (e.g. water supply) | RBMPs, water management, metering and allocation, water demand management, increase water efficiency and reusing, protection of vulnerable species and habitats, pricing policies, etc. |
| DPSIR focus | Responses focus on Impacts | Responses focus on Drivers, Pressures, and Impacts |
| Possible policy responses | Development of Drought Management Plans (DMPs), water allocation systems and water governance rules and regulations. Support for insurance systems. Financial support for emergency actions (e.g. Solidarity Funds) | Reduction of pressures via sectoral policies (e.g. agriculture, energy, urban development) related to water usage (e.g. on water-usage planning, water allocation and pricing systems, control) |

# Proposed working definitions

The following definitions could facilitate a common understanding of droughts and water scarcity by the European Commission and the Member States, if endorsed by the Water Directors:

## Drought definition

Drought is a natural phenomenon. It is a temporary, negative and severe deviation along a significant time period and over a large region from average precipitation values (a rainfall deficit), which might lead to meteorological, agricultural, hydrological and socioeconomic drought, depending on its severity and duration.

## Water scarcity definition

Water scarcity is a man-made phenomenon. It is a recurrent imbalance that arises from an overuse of water resources, caused by consumption being significantly higher than the natural renewable availability. Water scarcity can be aggravated by water pollution (reducing the suitability for different water uses), and during drought episodes.

## Complementary text

Both, the drought and the water scarcity definitions can be complemented by information on their impacts (based on Xerochore, 2007 and CONHAZ, 2012), whenever the context is appropriate (e.g. policy documents):

If droughts or water scarcity pass certain thresholds, they can significantly affect the environment (terrestrial and freshwater ecosystems, air, soils), the economy (agriculture and water uses) and society (e.g. public water supplies, welfare, recreational activities, cultural and aesthetic concerns).

# References

CIS (2007): Drought Management Plan Report – Including Agricultural, Drought Indicators and Climate Change Aspects. Technical Report - 2008 - 023

CIS (2009a): Guidance Document No. 20. Guidance Document on exemptions to the environmental objectives. Technical Report - 2009 – 027.

CIS (2009b): Guidance document No. 24. River basin management in a changing climate. Technical Report 2009 – 040.

CONHAZ (2012): Costs of Natural Hazards - A Synthesis. www.conhaz.org (accessed on March 2012)

European Commission (2007) Communication from the Commission to the European Parliament and the Council - Addressing the challenge of water scarcity and droughts in the European Union {SEC(2007) 993} {SEC(2007) 996}/* COM/2007/0414 final */

EEA (2001): Sustainable water use in Europe. Part 3: Extreme hydrological events: floods and droughts. EEA Environmental issue report 21, Copenhagen

EEA (2009): Water Resources across Europe-confronting water scarcity and drought. EEA Report Nº2/2009. ISSN 1725-9177. Copenhagen.

Informal meeting of Water Directors of the European Union, Candidate and EFTA Countries (Salzburg, 1/2 June 2006). Final Synthesis. Agenda item 8: Water scarcity – Document: Water Scarcity Management in the Context of WFD. Policy summary.

Grigg, N. S., E. C. Vlachos. 1990. Drought Water Management. Proceedings of a National Workshop held in Washington, D. C. November 1-2, 1988. Colorado State University

MED Joint Process WFD/EUWI Water Scarcity Drafting Group (2006): Water Scarcity Management in the Context of WFD.

Mishra & Singh (2010): A review of drought concepts. Journal of Hydrology 391: 202-216

Rossi G. (2000) Drought mitigation measures: a comprehensive framework", in J-Vogt and F.Somma (eds) Drought and Drought Mitigation in Europe, Kluwer , Dordrecht

Vogt, J. & Somma, F. (2000) (Eds.): Drought and Drought Mitigation in Europe. Kluwer Academic Publishers, 325 pp.

Xerochore (2007). Background Document II (Deliverable 5.1.). Synthesis report on Management and Policy Options for Conference on Drought

http://edo.jrc.ec.europa.eu/php/index.php?action=view&id=2

Based on Hennessy K. et al quoted in Xerochore (2007).

E.g. Mishra & Singh (2010).

Based on a figure attributed to Vlachos (1982) quoted in MED Joint Process WFD /EUWI Water Scarcity Drafting Group (2006) and attributable to Grigg & Vlachos (1990).

Aridity and desertification are not issues which are discussed in the Water Scarcity and Drought Expert Group

Sources: compiled from all quoted references

Note that the case of non permanent rivers should be carefully tackled.